# December 8, 2023

| Rank | Magnitude | Date | Start | Max | End |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | X2.25 | 4/20/2022 | 03:41 | 03:57 | 04:04 |  |
| 2 | X1.5 | 5/10/2022 | 13:50 | 13:55 | 13:59 |  |
| 3 | X1.3 | 3/30/2022 | 17:21 | 17:37 | 17:46 |  |
| 4 | X1.1 | 5/3/2022 | 13:08 | 13:25 | 13:31 |  |
| 5 | X1.1 | 4/30/2022 | 13:37 | 13:47 | 13:52 |  |
| 6 | X1.1 | 4/17/2022 | 03:17 | 03:34 | 03:51 |  |
| 7 | X1 | 10/2/2022 | 19:53 | 20:25 | 20:34 |  |
| 8 | M9.7 | 4/21/2022 | 01:47 | 01:59 | 02:05 |  |
| 9 | M9.67 | 3/31/2022 | 18:17 | 18:35 | 18:45 |  |
| 10 | M8.77 | 10/2/2022 | 02:08 | 02:21 | 02:26 |  |

https://www.spaceweatherlive.com/en/solar-activity/top-50-solar-flares/year/2022.html

Using new files provided by Bob Gerzoff

Layout:

Data Mapping for WSPR

| Column Name | Source field from database (starts with 0) – See Appendix A | Destination field (starts with 1) |
| --- | --- | --- |
| datetime | (0) datetime (UTC) | (1) Datetime (UTC) |
|  |  | Format YYYY-MM-DDThh:mm:ss |
| Tcall | (7) Callsign (sender) | (2) transmitting call |
| Tgrid | (8) sender grid | (3) transmitting grid |
| Tlat | Calculate from grid | (4) transmitting latitude |
| Tlang | Calculate from grid | (5)transmitting longitude |
| Tgridsrc | W | (6)transmitting source of grid info |
| Rcall | (3) Callsign of Reporter | (7) receiving call |
| Rgrid | (4) reporter grid | (8) receiving grid |
| Rlat | Calculate from grid | (9) receiving latitude |
| Rlon | Calculate from grid | (10) receiving longitude |
| Rgridsrc | W | (11) receiving source of grid info |
| Freq | (6) Mhz X 1,000,000 | (12) frequency (Hz) |
| SNR | (5) db | (13) SNR |
| Mode | W | (14) mode |
|  | W | (15) source |
|  | null | (16) sender Status |
|  | null | (17) iMD |
|  | null | (18) ipOriginID |
|  | null | (19) senderMobileLocator |
|  | null | (20) c1 |
|  | null | (21) c2 |
|  | W | (22) mode2 |
|  | (11) distance | (23)path length (km) assuming short path |
|  |  | Computed fields: |
|  |  | (24)latitude of signal center point |
|  |  | (25)Longitude of signal center point |

Summary of file processing:

“Datetime”,”Tcall”,”Tgrid”,“Tlat”,“Tlang”,“Tgridsrc”,“Rcall”,“Rgrid”,“Rlat”,“Rlon’,“Rgridsrc”,“Freq”,“SNR”,“Mode”

from csv import reader

import csv

import datetime

import re

outf = open('C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v2\\NorthAmerica.csv','at', newline='')

write=csv.writer(outf)

indir = 'C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v2\\'

infile = indir+"2022-03-30_WSPR.csv"

legal_gridsquares = ["CN","CM","DN","DL","DM","EN","EM","EL","FN","FM","FL"]

print(infile)

with open(infile,'r') as WSPRfile:

WSPRcsv = reader(WSPRfile)

for row in WSPRcsv:

Tgrid = row[2][0:4]

Rgrid = row[7][0:4]

Tgridpre = Tgrid[0:2]

Rgridpre = Rgrid[0:2]

if Tgridpre in  legal_gridsquares:

if Rgridpre in legal_gridsquares:

row.append(Tgrid)

row.append(Rgrid)

row=list(filter(None,row))

write.writerow(row)

WSPRfile.close()

outf.close()

print("Done")

fnNA <- "C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v2\\NorthAmerica.csv"

NAQSOS <- read.csv(fnNA)

names(NAQSOS) <- c("Datetime", "Tcall", "Tgrid", "Tlat", "Tlang", "Tgridsrc", "Rcall", "Rgrid", "Rlat", "Rlon", "Rgridsrc", "Freq", "SNR", "Mode","x1","x2","x3","x4","TG04","RG04")

NAQSOS$hrmin <- NAQSOS$hr + (NAQSOS$min/60)

meter40 <- NAQSOS[NAQSOS$band == 7, ]

NAQSOS$band <- trunc(NAQSOS$Freq/1000000)

SNRbyHR <- aggregate(NAQSOS$SNR, list(NAQSOS$hr), FUN = mean)

plot(SNRbyHR$Group.1, SNRbyHR$x)

plot(NAQSOS$hrmin, NAQSOS$SNR)

plot(meter40$hrmin, meter40$SNR)

## January 20 2024

### Cut data set by distance:

Line of Sight

meter40LT100m <- meter40[meter40$x2 >1 && meter40$x2 <100,]

NVIS 100 to 500 miles

meter40LT500m <- meter40[meter40$x2 >100 & meter40$x2 < 500,]

E-layer 500 to 1500 miles

meter40LT1500m <- meter40[meter40$x2 >500 & meter40$x2 < 1500,]

F-layer over 1500 miles

meter40GT1500m <- meter40[meter40$x2 > 1500,]

### Using lt 100 miles as a test:

Summarize SNR by Hour:

l100mSNRbyHR <- aggregate(meter40LT100m$SNR, list(meter40LT100m$hr), FUN = mean)

Plot:

ggplot(l100mSNRbyHR , aes(x = Group.1, y = x) ) +geom_line()

By tenth of an hour:

l100mSNRbyTenthHR <- aggregate(meter40LT100m$SNR, list(meter40LT100m$hrTenth), FUN = mean)

ggplot(l100mSNRbyTenthHR , aes(x = Group.1, y = x) ) +geom_line()

By quarter hour

meter40LT100m$qtrHR <- 15*trunc(meter40LT100m$min/15)+7

meter40LT100m$hrQtr <- meter40LT100m$hr+ (meter40LT100m$qtrHR/60)

l100mSNRbyqtrHR <- aggregate(meter40LT100m$SNR, list(meter40LT100m$hrQtr), FUN = mean)

ggplot(l100mSNRbyqtrHR , aes(x = Group.1, y = x) ) +geom_line()

And count of observations by hour:

l100mCNTbyHR <- aggregate(meter40LT100m$SNR, list(meter40LT100m$hr), FUN = length)

ggplot(l100mCNTbyHR , aes(x = Group.1, y = x) ) +geom_line()

### Now for distances over 1500 miles

meter40GT1500m <- meter40[meter40$x2 > 1500,]

GT1500mSNRbyHR <- aggregate(meter40GT1500m$SNR, list(meter40GT1500m$hr), FUN = mean)

ggplot(GT1500mSNRbyHR , aes(x = Group.1, y = x) ) +geom_line()

For WSPR obervatins over 1500 miles:

Overall decrease in SNR from hour before flare to hours after

| HR UTC |  |
| --- | --- |
| 14 | -9.732426 |
| 15 | -8.117530 |
| 16 | -9.176364 |
| 17 | -11.309051 |
| 18 | -12.776087 |
| 19 | -10.793939 |

Graph by Grid Square:

> l100mSNRbyGRID <- aggregate(meter40LT100m$SNR, list(meter40LT100m$RG04), FUN = mean)

> View(l100mSNRbyGRID)

> write.csv(l100mSNRbyGRID , "l100mSNRbyGRID .csv", row.names = FALSE)

From hour 16 to hour 17 was a drop of 2.13 Db SNR – formatted with Excel

| Time | SNR | Quarter Hour | Minutes of hour |
| --- | --- | --- | --- |
| 16.1166667 | -8.018634 | 1 | 7 |
| 16.3666667 | -9.793893 | 2 | 22 |
| 16.6166667 | -8.814159 | 3 | 37 |
| 16.8666667 | -10.186207 | 4 | 52 |
| 17.1166667 | -9.103448 | 1 | 7 |
| 17.3666667 | -10.426667 | 2 | 22 |
| 17.6166667 | -14.134615 | 3 | 37 |
| 17.8666667 | -16.103896 | 4 | 52 |
| 18.1166667 | -12.719697 | 1 | 7 |
| 18.3666667 | -13.491228 | 2 | 22 |
| 18.6166667 | -12.072072 | 3 | 37 |
| 18.8666667 | -12.815534 | 4 | 52 |
| 19.1166667 | -10.080745 | 1 | 7 |

Drop from 17:00 second quarter (conventionally listed as 22 minutes past the hour) and 17:00 third quarter is 3.71 Db SNR. From the second quarter to the 4th is 5.68 dB SNR.

### 500 to 1500 Miles

# Final Charts

# control data

from csv import reader

import csv

import datetime

import re

outf = open('C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v3-control\\May31.csv','at', newline='')

write=csv.writer(outf)

indir = 'C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v3-control\\'

infile = indir+"2022-03-31_WSPR.csv"

legal_gridsquares = ["CN","CM","DN","DL","DM","EN","EM","EL","FN","FM","FL"]

print(infile)

with open(infile,'r') as WSPRfile:

WSPRcsv = reader(WSPRfile)

for row in WSPRcsv:

Tgrid = row[2][0:4]

Rgrid = row[7][0:4]

Tgridpre = Tgrid[0:2]

Rgridpre = Rgrid[0:2]

if Tgridpre in  legal_gridsquares:

if Rgridpre in legal_gridsquares:

row.append(Tgrid)

row.append(Rgrid)

row=list(filter(None,row))

write.writerow(row)

WSPRfile.close()

outf.close()

print("Done")

Mar29fn <- "C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v3-control\\May29.csv"

May29 <- read.csv(Mar29fn)

names(May29) <- c("Datetime", "Tcall", "Tgrid", "Tlat", "Tlang", "Tgridsrc", "Rcall", "Rgrid", "Rlat", "Rlon", "Rgridsrc", "Freq", "SNR", "Mode","x1","x2","x3","x4","TG04","RG04")

Mar29 <- May29

rm(May29)

Mar31fn <- "C:\\Documents\\HAMSCIData\\WSPR data\\WSPR2022v3-control\\May31.csv"

Mar31 <- read.csv(Mar31fn)

names(Mar31) <- c("Datetime", "Tcall", "Tgrid", "Tlat", "Tlang", "Tgridsrc", "Rcall", "Rgrid", "Rlat", "Rlon", "Rgridsrc", "Freq", "SNR", "Mode","x1","x2","x3","x4","TG04","RG04")

Mar29$hr <- as.numeric(substring(Mar29$Datetime, nchar(Mar29$Datetime)-7, nchar(Mar29$Datetime)-6))

Mar29SNRbyHR <- aggregate(Mar29$SNR, list(Mar29$hr), FUN = mean)

Mar31$hr <- as.numeric(substring(Mar31$Datetime, nchar(Mar31$Datetime)-7, nchar(Mar31$Datetime)-6))

Mar31SNRbyHR <- aggregate(Mar31$SNR, list(Mar31$hr), FUN = mean)

ggplot(Mar31SNRbyHR , aes(x = Group.1, y = x) ) +geom_line()

ggplot(Mar29SNRbyHR , aes(x = Group.1, y = x) ) +geom_line()

names(Mar31SNRbyHR) <- c("Hour", "AvgSNR")

names(Mar29SNRbyHR) <- c("Hour", "AvgSNR")

Mar29COUNTbyHR <- aggregate(Mar29$hr, list(Mar29$hr), FUN = length)

names(Mar29COUNTbyHR) <- c("Hour", "Reports")

ggplot(Mar29COUNTbyHR , aes(x = Hour, y = Reports) ) +geom_line()+ggtitle("March 29 2022 - Nbr Observations per hour on 40 meters")

and for March 31

# Line of Sight

Mar29LT100m <- Mar29[Mar29$x2 >1 & Mar29$x2 <100,]

# NVIS 100 to 500 miles

Mar29LT500m <- Mar29[Mar29$x2 >100 & Mar29$x2 < 500,]

# E-layer 500 to 1500 miles

Mar29LT1500m <- Mar29[Mar29$x2 >500 & Mar29$x2 < 1500,]

# F-layer over 1500 miles

Mar29GT1500m <- Mar29[Mar29$x2 > 1500,]

# Line of Sight

Mar31LT100m <- Mar31[Mar31$x2 >1 & Mar31$x2 <100,]

# NVIS 100 to 500 miles

Mar31LT500m <- Mar31[Mar31$x2 >100 & Mar31$x2 < 500,]

# E-layer 500 to 1500 miles

Mar31LT1500m <- Mar31[Mar31$x2 >500 & Mar31$x2 < 1500,]

# F-layer over 1500 miles

Mar31GT1500m <- Mar31[Mar31$x2 > 1500,]

# Average SNR by hour, categorized by distance beween transmitter and receiver

Mar29LT100mSNRbyHR <- aggregate(Mar29LT100m$SNR, list(Mar29LT100m$hr), FUN = mean)

names(Mar29LT100mSNRbyHR) <- c("Hour", "AvgSNR")

Mar29LT500mSNRbyHR <- aggregate(Mar29LT500m$SNR, list(Mar29LT500m$hr), FUN = mean)

names(Mar29LT500mSNRbyHR) <- c("Hour", "AvgSNR")

Mar29LT1500mSNRbyHR <- aggregate(Mar29LT1500m$SNR, list(Mar29LT1500m$hr), FUN = mean)

names(Mar29LT1500mSNRbyHR) <- c("Hour", "AvgSNR")

Mar29GT1500mSNRbyHR <- aggregate(Mar29GT1500m$SNR, list(Mar29GT1500m$hr), FUN = mean)

names(Mar29GT1500mSNRbyHR) <- c("Hour", "AvgSNR")

Mar31LT100mSNRbyHR <- aggregate(Mar31LT100m$SNR, list(Mar31LT100m$hr), FUN = mean)

names(Mar31LT100mSNRbyHR) <- c("Hour", "AvgSNR")

Mar31LT500mSNRbyHR <- aggregate(Mar31LT500m$SNR, list(Mar31LT500m$hr), FUN = mean)

names(Mar31LT500mSNRbyHR) <- c("Hour", "AvgSNR")

Mar31LT1500mSNRbyHR <- aggregate(Mar31LT1500m$SNR, list(Mar31LT1500m$hr), FUN = mean)

names(Mar31LT1500mSNRbyHR) <- c("Hour", "AvgSNR")

Mar31GT1500mSNRbyHR <- aggregate(Mar31GT1500m$SNR, list(Mar31GT1500m$hr), FUN = mean)

names(Mar31GT1500mSNRbyHR) <- c("Hour", "AvgSNR")

# Add date of observation, to allow comparison in ggplot

Mar29LT100mSNRbyHR$dt <- "Mar29"

Mar29LT500mSNRbyHR$dt <- "Mar29"

Mar29LT1500mSNRbyHR$dt <- "Mar29"

Mar29GT1500mSNRbyHR$dt <- "Mar29"

Mar31LT100mSNRbyHR$dt <- "Mar31"

Mar31LT500mSNRbyHR$dt <- "Mar31"

Mar31LT1500mSNRbyHR$dt <- "Mar31"

Mar31GT1500mSNRbyHR$dt <- "Mar31"

Mar30SNRbyHR$dt <- "Mar30"

> Mar29SNRbyHR$dt <- "Mar29"

> Mar31SNRbyHR$dt <- "Mar31"

AllSNRbyHR <- rbind(Mar29SNRbyHR$dt, Mar30SNRbyHR, Mar31SNRbyHR )

> View(AllSNRbyHR) #oops

> AllSNRbyHR <- AllSNRbyHR[-c(1),]

> AllSNRbyHR <- rbind(AllSNRbyHR, Mar29SNRbyHR)

ggplot(data=AllSNRbyHR , aes(x=Hour, y=AvgSNR, group=dt, colour=factor(dt))) +

geom_line(size=.75) + geom_point()